���� � � � � ��������� � � � � �����������

���� ���� ���� ��� � �� ���� ���� ��������

������ �  �  ��� �� �� �� ���� ���� ������   VERSION 2.0

�������� ���� ���� �� � ��� ���� ���� ����

����������� � ��� � ��������� � � � � ����       FOR THE TI-85

DOCUMENTATION by Jason Todd <toddlers@erinet.com>

=============================================================================

INTRODUCTION

=============================================================================

Welcome to OS-85, an exciting new operating system for the Texas Instruments

TI-85 graphing calculator.  Similar to ZShell, this OS lets you run powerful

assembly language programs written for the Z80 microprocessor.  Programs

written exclusively for OS-85 will fit seamlessly into the environment, as if

they were part of OS-85.  To install this operating system on your TI-85,

just send the backup file (OS85V15.85B) to your calculator through a link

cable.  Or, get the backup from another TI-85.

Notes: OS-85 2.0 is compatible with ZShell 4.0, and can run anything and

everything written for ZShell.  Only programs which reference

undocumented memory addresses specific to ZShell will not

work right under OS-85.  I've only seen one such program,

noted below.

There is one small "catch" I think I should mention:  Do not try to

run a copy of Insane Game version 2.5 and earlier from OS-85,

because it will cause your calculator to crash when it tries

to exit.  Instead, run a newer version of the game, and it

will work just fine.  To make things easier, I've included a

copy of Insane Game version 2.7 in the original OS-85 2.0

release ZIP file.

As of right now, OS-85 ignores the ZS_BITS value.  It doesn't do

anything with checksums, and "warp out" isn't included.  I'll

definitely include checksum code in a later release (soon).

The warp out, however, may be part of a "registered" version

of OS-85.

=============================================================================

REQUIREMENTS

=============================================================================

OS-85 version 1.5 requires a TI-85 with ROM version 3.0A, 4.0, 5.0, 6.0, 8.0,

9.0, or 10.0.  The program string itself takes under 4K of free RAM.  Before

installing OS-85, you should check which ROM version is on your calculator.

To do so from the home screen press [2nd], [MODE], [ALPHA], [S].  You will

see 'Enter self test?' and a number below it.  DO NOT PRESS ENTER!  If you

press ENTER, your memory will be cleared.  The number you see is your ROM

version number.

=============================================================================

GETTING STARTED

=============================================================================

To run OS-85, press [CUSTOM], then [F1].  Depending on your setup, you will

either see the "OS-85 Home Screen" or one of your OS-85 programs executing.

If you've just received OS-85, you'll probably be at the OS-85 Home Screen.

Some special keys to remember in OS-85 are:

[F1]    Toggles between the main menu and the OS-85 Home Screen

[EXIT]  Lets you back out of an area within OS-85, or completely exit

if you are at the OS-85 Home Screen.

2x[2nd] Quick Powerdown, instantly shuts off the calculator.  When

you press [ON], you will be right back where you were.  Note

that you need to press this twice in a row for it to work.

[GRAPH] Puts a replica of the screen into the Graph memory.  From

there, you can change it or save it as a PIC.  Note that this

will only work if your graph is "clean."

=============================================================================

REVISION HISTORY

=============================================================================

1.0     I was just fooling around after learning Z80 assembly language, and

created version 1.0, which was originally titled "Windows 85," but I

didn't want any trouble from anyone over copyrights and such, so that

name didn't stick around very long.  This ran under ZShell.

1.1     A few minor changes to 1.0, also ran under ZShell

1.5     My first full-fledged, stand-alone beta release.  It took up 4,795

bytes of RAM, and had a lot of features that were somewhat pointless

or annoying.

2.0     This here little gem.  I reduced/reused/recycled the code throughout

OS-85, and made it a little more user-friendly.  Through these

optimizations, I was able to bring it down to 3,749 bytes.  That's a

decrease of 22% its original size!  New: LOADSTR feature--if you name

a string LOADSTR, it will automatically execute BEFORE OS-85 loads.

=============================================================================

PROGRAMMING FOR OS-85

=============================================================================

Programming for OS-85 is the same as programming for ZShell, with a few handy

additions to the TI-85.H include file.  These additions are described in

detail below.  Note that in the future, I hope to include such routines as

"OS_DRAWBOX", and "OS_MENUCURSOR."  These are used within OS-85, but I need

to work on the calling methods (the Inputs) for both of them.

MEMORY LOCATIONS:

(OS_VER) = ($8C41) = OS-85 Version number.  $15 = 1.5, a Beta release

(CURSORCHAR) = ($8C42) = Character for the OS-85 cursor (used for

OS_GETKEY).  If you set this to a value other than 00, than

that character will appear at the cursor position ($800C)

when OS_GETKEY is called.

CALL ADDRESSES:         Use CALL to use these.  Ex:  CALL OS_INPSTR

OS_GETKEY:  Waits for and returns a scancode.  This will intercept

[2nd] (Powerdown) and [GRAPH] (Copy to Graph), although it

will still return the scancodes for those keys.

INPUT:  None

OUTPUT:  A = Scancode

OS_DRAWSCREEN:  Redraws the screen (background PIC and menu bar)

INPUT:  None

OUTPUT:  HL,BC,DE are changed

OS_MENUBAR:  Clears the screen and draws the menu bar

INPUT:  None

OUTPUT:  HL,BC,DE are changed

OS_INPSTR:  Inputs a line of text up to 20 characters long, all

uppercase letters.  Text is stored as a length-terminated

string in the DELc buffer ($800F).

INPUT:  Set ($800C) to the screen row of input

OUTPUT:  ($800F) is length-terminated string

OS_INFOWINDOW:  Displays an information window.  To call, point HL to

an information data area, as follows:

(HL)    Width of title string / 8

(HL+1)  ASCIIZ title string

:  :   ASCIIZ Text line 1

:  :   ASCIIZ Text line X  (5 text lines maximum)

:  :   00  <- End of data area

HL will be incremented to the next byte after the data area

OS_FKEYWINDOW:  This is similar to OS_INFOWINDOW, but this displays a

menu for [F1]-[F5].  The calling method is the same for

OS_INFOWINDOW, but an Fkey for each text line will appear.

OS_POWERDOWN:  This will shut off the calculator.  Pressing ON will

turn it back on.

INPUT:  None

OUTPUT:  None

OS_PAUSE:  This will wait for the [ENTER] key to be pressed.

INPUT:  None

OUTPUT:  A = 9, for [ENTER]

VERY IMPORTANT:  It is a good idea to check (OS_VER), to make sure OS-85 is

in fact running.  If you try to call an OS_ procedure under ZShell,

the results would be unpredictable (the TI-85 will probably just

crash).